# Module 1 Continuous-time Signals

## Signals

A signal is something that carries information. Signals are usually physical quantities such as voltages or digital representations of such. For example, a speech signal is the changing air pressure from a vocal tract in time. A speech signal captured using a microphone is changing voltage in time. Figure 1 shows a continuous-time continuous-valued (analog) signal that is a single tone of 100Hz, .

Figure 1. Continuous-time Continuous-valued (Analog) Signal

Figure 2 shows a continuous-time discrete valued signal obtained by quantizing the signal shown in Figure 1 (representing using a finite number of bits). The step size for the quantized signal of Figure 2 is 0.25.

Figure 2. Continuous-time Discrete-Valued Signal

Figure 3 shows a discrete-time continuous-valued (discrete-time) signal obtained by sampling the signal shown in Figure 1. The sampling rate is eight points per period. Notice that for a discrete-time signal, the signal is only defined at the sample times. Discrete-time signals are sequences of numbers, and those numbers have infinite precision.

Figure 4 shows a discrete-time discrete-valued (digital) signal obtained by sampling and quantizing the signal shown in Figure 1. The sampling rate is eight points per period and the step size is 0.25. Like discrete-time signals, digital signals are sequences of numbers.

Figure 3. Discrete-time Continuous-valued (Discrete-time) Signal

Figure 4. Discrete-time Discrete-valued (Digital) Signal

Although many signals have time as the independent variable, some signals have other independent variables such as position. Still images, for example, are two dimensional functions of position  and are commonly represented as two- or three-dimensional arrays.

Signal type summary:

Analog signal: continuous-time and continuous valued, represented by continuous-time function/formula.

Discrete-time signal: discrete-time and continuous valued, represented by a sequence of numbers.

Digital signal: discrete-time and values come from discrete set of values, represented by a sequence of numbers that can be represented on a computer system.

## Continuous-time Signals

Physical world signals are analog signals. Physical signals (real world) are often very complex,  making it difficult to represent them exactly with a function. Complex signals may be with a function that closely approximates the signal.

Some of the more commonly encountered signal types include: impulses, steps, pulses, exponentials, sinusoids, and combinations of these.

## Impulse

The continuous-time unit impulse denoted  is a signal concentrated at one point with an area of one.

,  and

The unit impulse is not a function, and one cannot generate a unit impulse as it is a pulse with no width that is infinitely high. A unit impulse can be approximated by a very narrow and very high pulse with an area of one.

An approximation of an impulse is shown in Figure 5. The impulse is 0.01 seconds wide and thus has a height of 100 to give an area of one.

Figure 5. Continuous-time Impulse

## Step

The continuous-time unit step denoted  is a signal that is zero for t less than zero and 1 for t greater than zero.

The unit step has a discontinuity at t = 0. Practically, an ideal unit step cannot be created as signals must have a non-zero rise time. The unit step is shown in Figure 6.

The derivative of a unit step is a unit impulse and the integral of an impulse with upper limit of t is a unit step.

Figure 6. Unit Step

## Pulse

A continuous-time pulse signal is a signal that is one for t between zero and the pulse end and zero elsewhere. A pulse can be created using a difference of a step and a delayed step.

A pulse signal has a discontinuity at the start and end of the pulse. A pulse with a duration of one second is shown in Figure 7.

Figure 7. Unit Pulse with One Second Duration

## Exponential

Exponential signals have the form . The most common exponential signal typically encountered has r equal to Euler’s number (e). Exponential signals are often characterized in terms of their rate of increase or decrease (decay).

The signal  where a > 0 is a decreasing exponential for t > 0. The time constant of the exponential is t = 1/a. At one time constant, t =  t = 1/a, the signal has decreased by 1/e (to around 37% of the initial value) and by five time constants the signal has decreased to less than 1% of the initial value. Figure 8 shows an exponential signal with a = 2. The signal has a time constant of . The end time of the plot, 2s, corresponds to four time constants of the signal.

Figure 8. Exponential Signal with Time Constant 0.5 s

## Sinusoidal

A sinusoidal signal (cosine, sine) with a linear angle function has the form , where A is the amplitude,  is the radian frequency (radians/sec), and  is the phase (radians). The sinusoidal signal  shown in Figure 9 has an amplitude of 1.5 and oscillates between -1.5 and -1.5 every . The radian frequency of the signal is  rad/s or Hz. The signal has a phase shift of  radians.

Figure 9. Sinusoidal Signal with Amplitude 1.5 and Period 0.2 s

## One-sided, Two-sided, Finite Duration Signals

A two-sided signal is non-zero for all time, . The sinusoidal signal  shown in Figure 9 is a two-sided signal.  A signal that is multiplied by a step is a one-sided signal and a signal multiplied by a pulse is a finite duration signal. A right-sided signal is nonzero for  and is zero otherwise. A left-sided signal is nonzero for  and is zero otherwise.

The signal  is a right-sided signal that is non-zero for  and the signal  is a finite duration signal that is only non-zero for .

Figure 10. Right-Sided Signal

Figure 11. Finite Duration Signal

## Signal Properties

### Continuous Signals

A continuous-time signal  is continuous at a time  if  for an infinitesimally small value of. A signal that is continuous for all time is called a continuous signal (different meaning than continuous-time signal).

A signal  is discontinuous at a time  if  for an infinitesimally small value  (signal has a jump at time ). The unit step  has a discontinuity at time ;  and .

A signal that has a finite number of discontinuities (is continuous except at a countable number of times) is a piecewise continuous signal. The step is a piecewise continuous signal.

### Periodic Signals

A continuous-time signal  is periodic if there is a value T such that  for all time t. The fundamental period  is the smallest value of T for which . Sinusoidal signals (cos, sin, complex exponential) are periodic signals as are square waves (pulse trains). A signal that is not periodic is aperiodic.

The continuous-time signal  is sinusoidal with frequency  rad/s (10 Hz). The signal  repeats itself every ; it also repeats itself every 0.2s, every 0.3s, etc., but the smallest period of repetition is .

### Even and Odd Functions

A function  is even if . Even functions are symmetric about the y-axis. A function  is odd if . The cosine function is an example of an even function, and the sine function is an example of an odd function.

A signal can be expressed as a sum of even and odd components:

Where  is the even portion and  is the odd portion.

The sum or difference of even functions is also an even function. The sum or difference of odd functions is also an odd function. The product or quotient of two even or two odd functions is an even function. The product or quotient of an even function and an odd function is an odd function.

### Sampling Property of Impulse

A function multiplied by an impulse at  is an impulse at  scaled by the function evaluated at , .

For example, a right-sided exponential  multiplied by a delayed impulse  gives

### Signal Energy and Power

The energy E of a signal x(t) is

For the signal energy to be finite, the signal amplitude must approach 0 as  and . Generally, this means, the signal is either finite-duration or one sided and decreasing.

The signal power is the time average of the signal energy

The signal power is the square of the signal rms value. For the signal power to be finite, the signal must be periodic or statistically regular.

A signal that has finite energy is an energy signal and a signal that has finite and non-zero power is a power signal.

Note the energy and power here are measures of signal size and not the actual signal energy or power as they ignore the effect of the load. In addition, the units of energy and power here are not consistent with the typical units of energy and power but for signal comparisons the notion of energy and power here is useful.

## Signal Operations

### Time Shifting

A time shifted continuous-time signal is shifted either to the left (advanced) or right (delayed) in time. A shifted impulse, , has an area of one concentrated at  rather than 0. A shifted unit step, , is defined by

For example,  is a step shifted right (delayed) by 5 seconds and  is a step shifted to the left (advanced) by 2.5 seconds.

The signal  is a finite duration exponential delayed by 1 second and the signal  is a finite duration exponential advanced by 2 seconds.

Figure 12. Delayed Signal

Figure 13. Advanced Signal

### Time Scaling

A time scaled signal is either sped up or slowed down in time. The signal  for a > 0 is a time scaled version of . If a > 1 the signal is time compressed (sped up in time) and if a < 1 the signal is time expanded (slowed down in time). In the expanded signal of Figure 15, the nonzero portion is longer than the original signal of Figure 14 and in the compressed signal of Figure 16, the nonzero portion is shorter than the original signal of Figure 14.

Figure 14. Delayed Pulse Signal

Figure 15. Time Expanded Signal (a = 0.5)

Figure 16. Time Compressed Signal (a = 3)

### Time Reversal

The signal  is a time reversed version of . Graphically, time reversal corresponds to flipping the signal about the vertical axis. In the time reversed signal of Figure 18, the signal is flipped about the vertical axis compared to the original signal of Figure 17.

Figure 17. Delayed Finite-Duration Exponential Signal

Figure 18. Time-Reversed Signal

## Sketching and Plotting Signals

To accurately sketch or plot a signal, identify an appropriate time range to show the behavior of interest and how rapidly the signal changes. This will allow you to determine an appropriate time range and interval between points.

To sketch a sinusoid:

Determine the amplitude, this gives the height of positive and negative peaks.

Determine the period, this gives the time for one repetition.

Find the location of a positive peak, for a cosine this is where the angle function equals zero.

Positive and negative peaks are spaced  apart and zero crossings are  before and after peaks.

For plotting sinusoids, generally around 20 to 25 points are needed in each period for the signal to look smooth.

To sketch an exponential:

Determine the initial value at t = 0.

Determine the time constant.

In one time constant, the signal will have decreased by 1/e (or increased by e).

In three time constants, the signal will have decreased by 1/e3 (or increased by e3). In five time constants, the signal will have decreased to zero (or increased by e5, around 150 times).

The signal  is a right-sided sinusoid whose envelope decreases according to the time constant of the exponential portion of the signal. The fundamental period of the sinusoidal portion is T = 1/25 = 0.04s and the exponential time constant is t = 1/5 = 0.2s. The plot of Figure 19 shows three time constants of the signal (the sinusoid oscillates more rapidly than the exponential decay time constant).

Figure 19. Sinusoidal Signal with Decreasing Envelope

clear; clc; close all;

sympref('HeavisideAtOrigin', 1.0);

tau = 1/5;

T0 = 1/25;

t = -0.1:T0/25:3*tau;

x = 2*exp(-5*t).*cos(50*pi*t).*heaviside(t);

fig = figure;

plot(t,x,'k-','linewidth',1), grid;

axis([min(t), max(t), min(x)*1.05, max(x)*1.05])

xlabel('t (s)'),ylabel('x(t)');

Figure 20. MATLAB Code to Plot Signal of Figure 19

Last modified Wednesday, December 16, 2024

This work is licensed under CC BY-NC-SA 4.0. To view a copy of this license, visit https://creativecommons.org/licenses/by-nc-sa/4.0/