# Module 1 Sinusoidal Signals

## Sinusoidal Signals

The general form of a continuous-time sinusoid (cosine) with a linear angle function is , where A is the amplitude,  is the radian frequency (radians/second), and  is the phase (radians). Sinusoidal signals are important in communications; multimedia such as audio, images, and video; and signal processing.

The sinusoidal signal  has an amplitude of 3 and oscillates between +3 and -3 every T = 0.04s. The radian frequency of the signal is  rad/s or Hz. The signal has a phase shift of  radians which corresponds to a time shift of s or 5ms. The MATLAB code segment of Figure 1b was used to generate the plot of this signal shown in Figure 1a.

Figure 1a. Cosine with Amplitude 3, Frequency 25Hz, and Phase Shift -pi/4

% cosine with amplitude 3, frequency 25Hz, and phase shift -pi/4

f = 25; T = 1/f;

t = 0.0 : T/25 : 2*T;

x = 3*cos(2*pi*25*t - pi/4);

plot(t,x,'-k','linewidth',1), grid;

xlabel('t (s)'); ylabel('x(t)');

Figure 1b. MATLAB Code to Plot Cosine Signal

## Complex Exponential Signals

The general form of a complex exponential signal is  where A is the amplitude,  is the radian frequency, and  is the phase shift. Cosines and sines can be expressed in terms of complex exponentials using the Inverse Euler formulas,  and  , and complex exponential signals can be expressed in terms of cosines and sines using the Euler formula, .

The following signals are equivalent. The MATLAB code of Figure 2 shows how to compute the sinusoidal signal each way.

% cosine with amplitude 4, frequency 500Hz, and phase shift pi/2

f = 500; T = 1/f;

t = 0.0 : T/25 : 2*T;

xa = 4*cos(2*pi*500*t + pi/2);

xb = 2*exp(1j*(2*pi*500*t+pi/2))+2*exp(-1j*(2*pi*500*t+pi/2));

xc = real(4*exp(1j*(2*pi*500*t + pi/2)));

Figure 2. MATLAB Code to Compute Sinusoidal Signal

Complex exponential signals are commonly used instead of cosines and sines when analyzing signals in terms of their spectrum and for numerical computations.

## Complex Amplitude, Phasors

Complex exponential signals can also be expressed in terms of a complex amplitude and a sinusoidal part.

where

X is the complex amplitude (constant) and  is a sinusoid with an amplitude of one and a frequency of .

The complex exponential signal  has complex amplitude . The complex amplitude of a sinusoid is also referred to as the phasor.

Complex exponential signals and complex amplitudes (phasors) allow one to work with exponential functions which typically are easier to work with mathematically than trigonometric functions such as cosines and sines.  The complex amplitude is used when analyzing signals in terms of their spectrum.

Multiplication of phasors is done as multiplying exponentials. The magnitude of the result is the two magnitudes multiplied together and the angle of the result is the two angles added together. Adding phasors cannot be done directly. The phasors first are converted to rectangular form, added, and then converted back to phasors.

## Sketching and Plotting Sinusoids

To sketch a sinusoid, for example :

Determine the amplitude, this gives the height of positive and negative peaks. For this example, the amplitude A = 2.8.

Determine the period. For this example, the period is s.

Determine the location of a positive peak. For a cosine this is where the angle function equals zero. For this example, the angle function  and this is zero for  or 1.67ms.

The positive and negative peaks are spaced T/2 apart. For this example, the positive and negative peaks are 0.833ms apart.

The zero crossings are T/4 before and after peaks. For this example, the zero crossings are positive and negative peaks are 0.416ms from the peaks.

Sinusoids are typically plotted using software such as MATLAB. A plot of the signal  is shown in Figure 3a. The MATLAB code of Figure 3b created the plot. Notice that the peaks in Figure 3a have heights of 2.8 or -2.8, the positive and negative peaks are separated by T/2 = 0.833ms, and the zero crossings are T/4 = 0.416ms away from the peaks.

Figure 3a. Plot of Sinusoidal Signal

% cosine with amplitude 2.8, frequency 100Hz, phase shift -pi/3

f = 100; T = 1/f;

t = 0.0:T/25:2*T;

x = 2.8*cos(200*pi*t-pi/3);

fig = figure;

plot(t,x,'-k','linewidth',1), grid;

xlabel('t (s)'); ylabel('x(t)');

Figure 3b. MATLAB Code to Generate and Plot Continuous-time Sinusoidal Signal

For the plot of a sinusoidal signal to look smooth using software such as MATLAB, there needs to be around 25 points per period of the highest frequency sinusoid in the signal. MATLAB’s plot function connects the discrete points with lines and the peaks of sinusoidal signals look like triangles when using fewer points per period.

## Determining the Mathematical Representation of Sinusoid from a Plot

To determine the mathematical representation (or approximate representation) of a sinusoidal signal from a plot:

- Measure the height of a positive peak, this is the amplitude A.

- Measure the time between positive peaks (or negative peaks), this is the period T of the signal.

- The frequency of the signal can be determined from the period,  or .

Measure the time of the first positive peak with respect to t = 0, this is the time shift of the signal assuming the signal is a cosine.

The phase shift can be determined from the time shift, .

Figure 4. Sinusoidal Signal

For the signal shown in Figure 4:

The amplitude is approximately A = 3.5 (actual is 3.6).

The period is approximately T = 5ms (actual is T = 5ms) which corresponds to a frequency 200Hz.

The time shift is approximately td = 0.4ms (actual is 0.416ms) which corresponds to a phase shift of -p/6 rad; .

This method of determining the formula of a sinusoidal signal from a plot is only practical for a signal that is a single sinusoid and the plot has at least one full period of the signal. The determined phase shift is not unique as the same plot would correspond to a phase shift of  for an integer k.

## Which Signal Representation to Use

- The cosine (or sin) form of sinusoidal signals is used for determining the one-sided spectrum of signals. It is also used for numerically evaluating the formula for a signal and plotting the signal. The complex exponential form of sinusoidal signals is used for determining the two-sided spectrum and is also used for numerically evaluating the formula for a signal and plotting the signal. Exponential functions are computationally more efficient to evaluate than trigonometric functions.

Last modified Tuesday, January 7, 2025

This work is licensed under CC BY-NC-SA 4.0. To view a copy of this license, visit https://creativecommons.org/licenses/by-nc-sa/4.0/