# Module 3 Discrete-time System Response

## Discrete-time System Response

The procedure for determining the system response of a LTID system is similar to that for continuous-time system (LTIC) response. The convolution sum is used in place of convolution integral for LTID systems.

The response or output  of a LTID system can be determined analytically using the convolution sum if the impulse response  of the system is known.

If the two functions being convolved are both zero for . i.e. both are right sided, then the convolution sum becomes

The lower limit of zero and upper limit of n is due to  and  for  (right sided).

## Properties of Convolution

Like the convolution integral, the convolution sum is commutative, associative, distributive over addition, and shifted inputs or impulse responses produce a corresponding shifted output.

The convolution sum being commutative means either of the following forms of the convolution sum can be used. For graphical convolution, it is often more straightforward to use the second form with the flipped shifted input.

Time shifted inputs or impulse responses produced a corresponding time-shifted output via convolution.

If

and

## Convolution with Impulses

The convolution sum of a function with an impulse yields the function and the convolution sum of a function with a shifted impulse yields the function shifted by the impulse shift.

## Graphical Convolution (Convolution Sum)

Similar to graphical convolution for the convolution integral, graphical convolution can be used to break the summation up into regions and solve the simpler summations. Graphical convolution is generally not necessary for discrete-time convolution as the convolution sum is generally fairly straightforward to evaluate although determining an analytical formula for the response is often not practical. Graphical convolution does allow one to restrict the limits of the summation for each piece rather than evaluating the full convolution sum.

For the form of the convolution sum below

and  are the input and impulse response with a change of variable ,  is the input flipped about the vertical axis, and  is the input flipped about the vertical axis and then shifted by n samples.

Consider response of a LTID system with impulse response  for the input . The convolution sum is

To identify the regions, the product of the functions for each region, and the limits of summation for each region, it is helpful to sketch a flipped shifted version of one of the functions and the stationary other function and then slide the flipped shifted function through the stationary function and seeing where they overlap and what the product of the functions is.

When there is no overlap,  in that region. When there is overlap (partial or full), the convolution sum must be evaluated for each region of overlap. The limits of the summation are determined by the region of overlap. The product of the two functions is summed.

Performing the change of variables on both  and  and flipping and shifting the input we obtain

and

There are five regions in the discrete-time convolution since both signals are finite duration and of different lengths.

Figure 1a shows the stationary function  and the flipped shifted function  for n = -2, this is also the image for the region of no overlap on the left. For the region of no overlap on the left, which is for , the output is .

Figure 1a. No Overlap on Left, n = -2

Figure 1b shows the flipped shifted input and stationary impulse response for n = 1. For the region of partial on the left, which is for , the output is . The non-zero product is from the left edge of the impulse response to the right edge of the flipped shifted input which gives the limits of the summation.

Figure 1b. Partial Overlap on the Left, n = 1

Figure 1c shows the flipped shifted input and stationary impulse response for n = 4. For the region of full overlap, which is for , the output is . The non-zero product is from the left edge of the impulse response to the right edge of the impulse response.

Figure 1c. Full Overlap, n = 4

Figure 1d shows the flipped shifted input and stationary impulse response for n = 6. For the region of partial overlap on the right, which is for , the output is . The non-zero product is from the left edge of the input to the right edge of the impulse response.

Figure 1d. Partial Overlap on the Right, n = 6

Figure 1e shows the flipped shifted input and stationary impulse response for n = 9. For the region of no overlap on the right, which is for , the output is . There is no non-zero product.

Figure 1e. No Overlap on the Right, n = 9

Combining the results from each region yields a piece-wise function for the response determined using graphical convolution.

- The response determined using a convolution sum table is generally a more useful form of the analytical response.

- Figure 2. Response Determined using Convolution Sum

## Regions for Discrete-time Graphical Convolution

Regions and summation limits for discrete-time convolution are identified in the same way as for continuous-time convolution. For two right-sided functions, there will be two regions in the graphical convolution: no overlap on left and partial overlap on the left. For two finite duration functions, there will be five regions (four if the durations are exactly the same) in the graphical convolution: no overlap on left, partial overlap on left, full overlap, partial overlap on right, and no overlap on right.

## Determining the Response using Convolution

The convolution sum can be evaluated directly, determined using convolution sum tables, or using graphical convolution. The impulse response of LTID systems is generally a sum of right-sided discrete-time exponentials or a sum of right-sided discrete-time exponentials multiplied by sinusoids.

For a weighted sum of impulses input, convolution with an impulse can be used.

For step, pulse, or right-sided exponential inputs, the convolution sum can usually be done using tables.

For inputs that are a combination of steps, pulses, or right-sided exponentials, the convolution can be done using convolution sum tables and superposition.

The convolution sum can be evaluated numerically.

For inputs that are not in the convolution sum table and would be difficult to evaluate the convolution sum directly, the convolution sum can be done using graphical convolution.

## Discrete-time System Response Examples

- Consider determining the response of a LTID system with impulse response  to a scaled step input .

- Since both the impulse response and input are right sided functions, there are two output regions: no overlap on left and partial overlap on left and the convolution sum for these functions can be evaluated directly.

The input will be 2.0 and the impulse response will be  where there is a non-zero product. The output will be zero for n <0 because the input is a step. The convolution sum is non-zero when there is partial overall on the left and the non-zero product is from the left edge of the impulse response to the right edge of the flipped shifted input.

- , n = 0, 1, 2, …

etc.

Notice that at each point n, the convolution sum gives the system response y[n] to the current input x[n] and the past inputs x[n-k] being held in system memory.

The two functions are both found in convolution sum tables, so the convolution sum could also be done using a convolution sum table. This will give a formula for the response which is often desirable. Using the convolution sum table for  and  and scaling

- The response is shown in Figure 3. The response from the convolution sum and the formula determined using the convolution sum is the same. The convolution sum if not determined using a table is evaluated numerically.

Figure 3. Response from Convolution Sum/Convolution Table

Consider determining the response of a LTID system with impulse response  to the input .

- Since both the impulse response and input are right sided functions, the convolution sum for these functions can be evaluated directly.

The input will be  and the impulse response will be  where there is a non-zero product. The output will be zero for n <0 because the input is right-sided and only non-zero for .

- , n = 0, 1, 2, …

- The convolution sum was evaluated using the MATLAB code of Figure 4. The response determined using the convolution sum is shown Figure 5. The convolution sum could be implemented using nested for loops (if the software does not have a built in convolution sum function), an inner loop for the summation over k and an outer loop for the range of n.

clear; clc; close all;

sympref('HeavisideAtOrigin', 1.0);

% response from convolution sum

hn = (5.0*(-0.1).^n).*heaviside(n);

xn = 2.0*heaviside(n) - (2.0*(0.8).^n).*heaviside(n);

yn = conv(hn,xn);

fig = figure;

stem(n,yn(1:length(n)),'-k'), ylabel('y[n]'), xlabel('n');

fig.Position = [680, 458, 400, 200];

axis([min(n), max(n), min(yn)*1.05, max(yn)*1.05])

- Figure 4. MATLAB Code for Evaluating Convolution Sum

- Figure 5. Response Determined using Convolution Sum

- The functions for the impulse response and both portions for the input are found in convolution sum tables, so the convolution sum can be done using a convolution sum table.

## Determining the Response using Difference Equation

The response of LTID systems can be determined analytically using convolution sum tables and graphical convolution although the result of graphical convolution is not always a straight-forward formula like determined using convolution sum tables. The response can be determined numerically using the convolution sum or the difference equation. The difference equation and convolution sum are equivalent for finite impulse response (FIR) systems, but the difference equation generally works better than the convolution sum for infinite impulse response (IIR) systems.

### Response of FIR Filters using Difference Equation

Since for a FIR filter the impulse response and filter coefficients are the same, convolution for a FIR is the same formula for computing the output response of a causal FIR filter using the difference equation.

Consider determining the response of a FIR filter with difference equation  for a unit step input. This is a third order (M=3) FIR filter with filter coefficients . The filter length is L = M+1 = 3. For a unit step input  and .

The step response determined using the difference equation is

and

MATLAB has a built in function filter that determines the response of a LTID system using the difference equation. The MATLAB code of Figure 6 determines the impulse and step response of the filter with difference equation .

clear; clc; close all;

sympref('HeavisideAtOrigin', 1.0);

% FIR filter coefficients

bk = [0.0, 0.5, 0.25, 0.25];

% impulse response

n = -2:1:15;

dn = zeros(1,length(n)); dn(n==0) = 1.0;

hn = filter(bk,1,dn);

% step response

xn = heaviside(n);

yn = filter(bk,1,xn);

Figure 6. MATLAB Code for Determining Response using Difference Equation (filter)

Figure 7. Impulse Response of FIR Filter Determined using Difference Equation

Figure 8. Step Response of FIR Filter Determined using Difference Equation

Notice that for a FIR filter, the impulse response, response for , is the same as the values of the filter coefficients,

### Response of IIR Filters using Difference Equation

IIR filters are linear and time-invariant if they met the initial rest conditions, a suddenly applied input at  and . For IIR filters, the impulse response is of infinite length and thus the convolution sum is an infinite summation. The convolution sum is only practical to use for IIR filters when formulas exist for both the impulse response and the input. The output of IIR filters is usually computed using the difference equation.

Consider determining the response of an IIR filter that meets the initial rest conditions with difference equation  for a unit step input. This is a second order (M=1, N=2) IIR filter with filter coefficients  and . The feedback coefficients for MATLAB come from the difference equation form where all the output terms are moved to the left hand side.

For an IIR filter with an input that has its first nonzero value at , determining the output  requires knowledge of past values of the output. The past values of the output needed to determine the output are the initial conditions of the system. If the system meets the initial rest conditions, the initial conditions are zero.

The step response determined using the difference equation is

After n = 7, the output oscillates between 0.833 and 0.167

The MATLAB code of Figure 9 determines the impulse and step response of the filter with difference equation .

Note that the output does not start until the input does. This is due to the filter being causal and meeting the initial rest conditions. Also note that the impulse response is infinite length due to the feedback terms in the difference equation (non-zero for ) although the impulse response values often approach 0 as n increases.

clear; clc; close all;

sympref('HeavisideAtOrigin', 1.0);

% IIR filter coefficients

bk = [0.75, -0.25, 0.25];

al = [1, 0.5, -0.5];

% impulse response

n = -2:1:15;

dn = zeros(1,length(n)); dn(n==0) = 1.0;

hn = filter(bk,al,dn);

% step response

xn = heaviside(n);

yn = filter(bk,al,xn);

Figure 9. MATLAB Code for Determining Response using Difference Equation

Figure 10. Impulse Response of IIR Filter Determined using Difference Equation

Figure 11. Step Response of IIR Filter Determined using Difference Equation

Last modified January 21, 2025

This work is licensed under CC BY-NC-SA 4.0. To view a copy of this license, visit https://creativecommons.org/licenses/by-nc-sa/4.0/