# Module 3 Discrete-time Systems

## Discrete-time Systems

A discrete-time system operates on a sequence of numbers and produces a sequence of numbers. Discrete-time systems implemented on a computer system with finite precision are digital systems. Digital systems are reliable and the processors are small and relatively cheap. The filtering algorithms are often easily modifiable.

System type summary:

- Analog systems process analog (continuous-time and continuous-valued) inputs and produce analog outputs. The signals processed or produced by continuous-time systems could be discrete-valued.

- Digital systems process digital (discrete-time and discrete valued) inputs and produce digital outputs. The signals processed or produced by discrete-time systems could be continuous-valued but in practice, discrete-time systems are realized implemented using computer systems so discrete-time and digital system are often used interchangeably.

## Discrete-time System Models

- As for continuous-time systems, discrete-time systems can be represented through relationships between the system variables. Input-output difference equations and convolution sum (impulse response) are time-domain models. Transfer functions are frequency domain (z or W domain) models.

The input-output difference equation, transfer function, and impulse response for a second order recursive discrete-time system are given below.

Difference equation

Transfer function

Impulse response

Discrete-time systems can be represented by:

Difference equations (time domain representation)

Convolution sum model, impulse response (time domain representation)

Frequency response (Discrete-time Fourier transfer representation, frequency domain representation)

Transfer function (z transform representation, frequency domain representation)

The frequency response and transfer function representations will be covered later.

## Discrete-time System Properties

The system properties of interest for continuous-time systems are also important for discrete-time systems.

### Linear Systems

A discrete-time system is linear if it is additive and homogeneous. For a linear system, if  and , then .

Like continuous-time systems, the response of a discrete-time linear system can be divided into two parts: the response due to the initial conditions (zero input response) and the response due to the input (zero state response).

A discrete-time delay system is linear.

,

### Time Invariant Systems

A discrete-time system is time-invariant if the system does not change as a function of time. For a time-invariant system, if  then the response to a shifted input  is . A system is time varying if it is not time invariant. Time varying systems are direct functions of time (for discrete-time systems n corresponds to time although the units are not seconds).

A discrete-time delay system is time-invariant.

### Causal and Memoryless Systems

A discrete-time system is causal (non-anticipatory) if the output  at some time  depends only on the input  for . In other words, if  for  then  for . A system is noncausal if it is not causal.

The output does not precede the input if the system is causal; i.e. first non-zero output sample occurs at same time or after first non-zero input sample. Noncausal systems can only be physically implemented in systems that are performing off line processing (all data already collected and processed off line of the physical process).

A discrete-time system is memoryless if the output at some time  only depends on the input at time .

A discrete-time pass through system  is causal and memoryless. A discrete-time delay system is causal but not memoryless. A discrete-time advance system  is non causal and not memoryless.

### Invertible Systems

A discrete-time system is invertible if the system input x[n] can be determined from the system output y[n].

The discrete-time system  is invertible. The system  would reconstruct the input from the output.

The discrete-time system (a two point averaging filter) is not invertible. For every possible y[n] value, there are an infinite number of x[n] and x[n-1] values that could have produced that average value.

### Stable Systems

A discrete-time system is bounded input bounded output (BIBO) stable if all bounded inputs produce bounded outputs. System stability for LTID systems (discrete-time linear time-invariant systems) is usually determined using the impulse response or transfer function.

## System Impulse Response

The impulse response of a system is the system response for the unit impulse input with zero initial conditions. The impulse response for a LTID system is denoted h[n].

The output  of a LTID system with impulse response  is the convolution sum of the impulse response with the input.

If one knows the impulse response of the system, the output response to any input can be determined using the convolution sum (assuming zero initial energy in the system).

The discrete-time delay system  has impulse response .

A discrete-time four point equally weighted average wit difference equation  has impulse response .

The impulse response of a causal LTID (linear time-invariant discrete-time) system is right sided and zero for time less than zero, .

The impulse response of a BIBO stable LTID system is absolutely summable.

Stable discrete-time systems have impulse responses that are of finite duration or are one-sided and decay to zero.

## Cascaded and Parallel Systems

More complex LTID systems can be constructed by cascading or connecting in parallel LTID systems. Figures 1a and 1b show cascaded and parallel combinations of two LTID systems.

Figure 1a. Cascaded LTID Systems

The overall impulse response of two cascaded LTID systems is the convolution of the two impulse responses, . The order of the two cascaded LTI systems does not matter for the overall impulse response (convolution is commutative).

Figure 1b. Parallel Connection of LTID Systems

The overall impulse response of a parallel combination of two LTID systems is the sum of the impulse responses, .

## Non-Recursive and Recursive and Difference Equations

The difference equations representing a LTID system can either be non-recursive or recursive. FIR (finite impulse response) filters have non-recursive difference equations representations and IIR (infinite impulse response) filters have recursive difference equation representations. FIR and IIR filters are commonly used in signal processing applications.

### Non-Recursive (FIR)

A causal FIR filter can be represented as a summation of filter weights multiplied by delayed input samples: for each n, . FIR filters are linear and time invariant.

The  are the filter coefficients (weights), the filter order is M, and the filter length is L = M+1. For a causal FIR, the summation index starts at k = 0 and the output  only depends upon the current and past input samples. The filter order corresponds to the largest delay.

A three-point averaging filter is an example of an order two FIR filter.

### Recursive (IIR)

A causal IIR filter can be represented as a summation of filter weights multiplied by delayed output samples and filter weights multiplied by delayed input samples: for each n,

Notice that the IIR system has feedback terms and is recursive. The prior output samples are used in the current output computation. The  coefficients are called the feedback coefficients and the  coefficients are the feed-forward coefficients. IIR systems have N+M+1 coefficients: N feedback coefficients and M feed forward coefficients . The order of a causal IIR system (maximum required delay) is the larger of N and M. One typically refers to the order of an IIR system as N. Notice that if the feedback coefficients are all zero (output only depends on the present and past values of the input) then the FIR system form is obtained.

A reverberation filter is an example of a IIR filter. The difference equation below is an order d+1 reverberation filter. The magnitude of the gains for the reverberation terms should be less than one.

If the system is initially at rest (output is zero prior to start time ,  for  ) and the input to the system is suddenly applied (input is zero prior to start time ,  for ) then the system meets the initial rest conditions. IIR filters that meet the initial rest conditions are LTI and can be characterized by their impulse response.

## Discrete-time System Examples

Consider a first order FIR . This system is causal but not memoryless.

,

, linear

, time invariant

A discrete-time modulator system  is neither linear nor time invariant. This system is causal and memoryless.

, not linear

, not time-invariant

## MATLAB System Models

Difference equations are represented in MATLAB by arrays of the difference equation coefficients from the rearranged form

With the a0 coefficient being 1. For an FIR filter, the a coefficients are a single 1.

Transfer functions of discrete-time systems are represented in MATLAB using the numerator and denominator polynomials. MATLAB supports symbolic representations of transfer functions. Discrete-time systems can also be modeled using Simulink.

Figure 2 shows MATLAB representations for the second order FIR filter with difference equation

and the second order IIR filter with difference equation

% second order FIR difference equation representation

b = [0.33, 0.33, 0.33];

a = 1;

% second order IIR difference equation representation

b = [0.0, 0.6, -0.2];

a = [1, 0.3, -0.1];

Figure 2. MATLAB Models for FIR and IIR Filters

Last modified January 21, 2025

This work is licensed under CC BY-NC-SA 4.0. To view a copy of this license, visit https://creativecommons.org/licenses/by-nc-sa/4.0/