# Module 6 Sampling

## Discrete-time Signals

Discrete-time signals have discrete time and continuous amplitude. Digital signals are discrete-time signals whose values belong to some finite set of values. Discrete-time and digital signals are represented by sequences of numbers x[n].

## Sampling and Sampled Signals

Sampling converts a continuous-time signal to a discrete-time signal which is then quantized to convert to a digital signal. Sampling involves periodically measuring a signal (think of reading the output of a switch that closes every so many seconds).  Sampling is used to convert analog physical world signals to discrete-time signals and is used extensively in telecommunications and signal processing.

Sampling the continuous-time signal x(t) every Ts seconds yields the discrete-time signal x[n], where x[n] is only defined at the integer indices n. The sampling frequency  is the number of samples taken per second and is the inverse of the sampling period. The units of sampling frequency are Hz (samples/second).

Figure 1. Block Diagram of Analog to Digital Converter

,

The discrete-time signal x[n] is a sequence of numbers and contains no information about the time or sampling rate. The discrete-time signal x[n] is only defined for integer n.

Consider sampling a continuous-time signal  using a sampling frequency of . The sampling period is so the signal is measured every 0.02s. Figure 2 shows the continuous-time signal and the samples using . The sampled signal is a sequence of numbers

Figure 2. Continuous-time Signal and Samples

## Sampling Theorem

Sampled signals originating from different continuous-time signals can be indistinguishable from each other (their sequences are identical). For a signal to be unambiguously reconstructed from its samples, one must sample at least two times highest frequency component (highest spectral component) in signal, . The minimum sampling rate is called the Shannon or Nyquist rate.

The necessary sampling rate depends upon the frequency content of the signal and the highest frequency component in the signal depends upon the application. Practically, the sampling rate needs to be slightly higher than twice the highest frequency to account for phase.

Some typical sampling frequencies are:

Biomedical applications, maximum frequency 1kHz, sampling frequency 2kHz

Audio (CD), maximum frequency 22kHz, sampling frequency 44.1kHz

Human speech, maximum frequency 4kHz, sampling frequency 8kHz

For the signal  to be unambiguously represented by its samples it would need to be sampled at . Figure 3a shows the continuous-time signal and the sampled signal using  and Figure 3b shows the continuous-time signal and the sampled signal using . In both cases the sampling frequency is high enough so the signal is unambiguously represented by its samples.

Figure 3a. Sampled Signal, fs = 600Hz

Figure 3b. Sampled Signal, fs = 2kHz

## Bandlimited Signals

For a signal to have a maximum frequency, it must be bandlimited, i.e. its spectrum must have a highest frequency component after which the spectrum is zero. A band limited signal  with maximum frequency ,  for , can be reconstructed from its samples if the signal is sampled at twice the highest frequency in the signal, .

Signals are often not band limited, any finite duration signal or periodic signal with discontinuities will not be bandlimited. Recall that the Fourier series for a square wave required an infinite number of terms (infinite spectrum).

One way to ensure only bandlimited signals are sampled is to remove all frequencies greater than , where  is the highest frequency the system was designed to handle,  .

Signals that are not band limited can be band limited using an anti-aliasing prefilter. The anti-aliasing filter must remove all the frequency components greater than , i.e. it must be an analog low pass filter with cutoff frequency  (and a sharp enough transition between pass and stop band).

## Sampling Examples

For the sinusoidal signal , the highest spectral component is  rad/s.  For the signal to be unambiguously represented by its samples the signal would need to be sampled at . If the signal is sampled at , the sampled signal is

For a periodic square wave signal with period

The Fourier series representation is

where  rad/s, , and ,

The signal is not bandlimited so there is no . The signal cannot be unambiguously represented by its samples. A signal only including the first N harmonics would have a maximum frequency , and this approximation of the signal could be unambiguously represented by its samples. For example, for N = 100, the approximation of the signal is

The maximum frequency is  rad/s. The approximated signal would need to be sampled at or higher.

## Oversampling

One could sample at very high rates, which would guarantee the sample rate was high enough, but this would generate more samples than necessary requiring extra storage and extra processing.

Consider CD audio which uses a sampling frequency 44.1kHz (44100 samples per second). The

Sampling frequency was chosen to capture the frequencies at the upper end of human hearing 18 – 20kHz. Practically, most people have a upper hearing limit of 12 – 16KHz due to age and hearing damage.

CD audio represents each sample using 16 bits (two bytes). Stereo audio requires two channels (left and right). One minute of CD audio requires over 10Mbytes of storage

Instead, consider sampling the audio signals using a much high rate like 1MHz. One minute of CD audio then requires

Songs are typically two to five minutes, so a single song would require 480Mbytes to 1.2Gbytes of storage each. Only one song could be stored on a standard CD.

## Discrete-time Spectrum

Discrete-time signals are typically analyzed in terms of their frequency content and discrete-time systems are typically analyzed and designed using frequency domain techniques just like for continuous-time signals and systems.

Sampling maps continuous-time frequencies  rad/s to discrete-time frequencies . The entire range of continuous-time frequencies  rad/s are mapped to the discrete-time frequencies   rad. The discrete-time frequency resulting from sampling is the continuous-time frequency divided by the sampling frequency. Discrete-time frequencies close to 0 correspond to low frequencies (slower oscillations) while those near  correspond to high frequencies. This discrete-time frequency has units of radians and is also called the normalized radian frequency or normalized frequency.

, where

The units of discrete-time frequency  are radians (unitless), thus the sampled signal does not contain any time information; it is just a sequence of numbers. Time information for the sampled signal can be determined if one knows the sampling frequency.

The variables  or  are both used to denote discrete-time frequency. In these materials,  is used for discrete-time frequency.

Consider determining the discrete-time spectrum of a continuous-time signal  sampled at . The two-sided continuous-time spectrum is

The corresponding discrete-time signa; and discrete-time spectrum resulting from sampling the signal at  are

and

,

The continuous-time and discrete-time spectrums are shown in Figures 4a and 4b respectively.

Figure 4a. Two-Sided Continuous-time Spectrum of Signal

Figure 4b. Two-Sided Discrete-time Spectrum of Sampled Signal

Last modified February 27, 2025

This work is licensed under CC BY-NC-SA 4.0. To view a copy of this license, visit https://creativecommons.org/licenses/by-nc-sa/4.0/