# Module 9 Transfer Functions

## Discrete-time Transfer Functions

The transfer function (system function) of a discrete-time system, typically denoted by , is the ratio of the transform of the output of the system to the transform of the input of the system with the initial conditions of the system set to zero.

, where Y(z) is the transform of the output and X(z) is the transform of the input.

Systems have a unique transfer function and the system’s transfer function completely characterizes the system behavior for zero initial conditions.

The transfer function is determined either from the difference equation describing the system or from the system’s impulse response.

The transfer function is the z-transform of the system impulse response.

The transfer function of a discrete-time system can also be determined from the system difference equation by taking the z-transform of both sides of the difference equation and solving for Y(z)/X(z).

For causal finite impulse response (FIR) filters, the general difference equation and transfer function representations are:

For causal infinite impulse response (IIR) filters, the general difference equation and transfer function representations are:

The system function H(z) is the ratio of a numerator polynomial B(z) with coefficients that come from the feed forward terms and a denominator polynomial A(z) with coefficients that come from the feedback terms. It is important to note that the B(z) polynomial is not the z-transform of output and the A(z) polynomial is not the z-transform of the input.

### Transfer Function Examples

The transfer function for a finite impulse response filter with impulse response  is . The corresponding difference equation is .

The transfer function for an infinite impulse response filter with impulse response  is . The corresponding difference equation is .

The transfer function for an IIR with difference equation

is

The impulse response of the system can be determined by taking the inverse z-transform of the transfer function.

## System Poles and Zeros

The zeros of a rational function of z, H(z), are the values of s satisfying H(z) = 0. The poles of a rational function of z, H(z), are the values of s satisfying H(z) = infinity.

For a rational function of z, , the zeros are the values of z satisfying B(z) = 0 and the poles are the values of z satisfying A(z) = 0, i.e. the zeros are the roots of the numerator polynomial and the poles are the roots of the denominator polynomial. If the degree of the numerator and denominator are not the same, the system will have either additional zeros at infinity or additional poles at zero so that the number of poles and zeros is the same and equal to the filter order.

For the 4-point moving average filter , the system function is  and the zeros and poles are

Zeros, ,

Poles, ,  (repeated three times)

FIR filters have all their poles at the origin, z = 0. The system function of an FIR system is determined by the zeros.

For the first order IIR filter , the system function is

Zeros, ,

Poles, ,

A pole-zero plot is a plot of the poles and zeros of a system in the complex z-plane, the x-axis is the real part of z and the y-axis the imaginary part of z. Evaluating over 0 to 2 traces a unit circle centered at the origin in the z-plane.

MATLAB has a roots function that can be used to find the poles and zeros of a system. Polynomials are represented in MATLAB by a vector of their coefficients. For the system , the MATLAB code of Figure 1 determines the poles and zeros of the system.

Bz = [1, 0.5];

Az = [1, -0.5];

z = roots(Bz);

p = roots(Az);

Figure 1. MATLAB Code to Determine System Zeros and Poles

## System Stability

Discrete-time systems are bounded-input bounded-output (BIBO) stable if all bounded inputs result in bounded outputs. For a bounded input to produce a bounded output, the impulse response of the discrete-time system must be of finite length or die out as n gets large, i.e. if h[n] approaches 0 as n approaches infinity then y[n] will be bounded whenever x[n] is bounded.

FIR systems have a finite length impulse response and therefore are BIBO stable. IIR systems may be stable or unstable. Consider a causal first order IIR system that meets the initial rest conditions

If  the impulse response decays to zero exponentially as , the convolution sum produces bounded output for bounded input, the system is stable.

If  the impulse response grows exponentially as , the convolution sum can produce unbounded outputs for bounded inputs, the system is unstable

If  the impulse response is bounded but infinite length, the convolution sum can produce unbounded outputs for bounded inputs, the system is unstable (this case is sometimes referred to as marginally stable since some bounded inputs will produce bounded outputs but not all)

Since  is the pole of the system, by looking at the system poles we can determine whether the system is stable or not. Given a system with a distinct pole at z = p, the impulse response will have a term of the form . The term will decay to zero as n gets large if the magnitude of p is less than one.

The following are equivalent tests for BIBO stability for a discrete-time system

- The impulse response  as .

- All the system poles have magnitudes less than one (this is equivalent to all the poles being inside the unit circle in the z-plane).

The impulse response and poles of the system indicate whether system is BIBO stable, a system’s stability does not depend upon the input to the system.

Consider the first order system , the system function is

. The system has a single pole at z = 0.5 so the system is stable. The impulse response and step response of the system are shown in Figures 2a and 2b.

Figure 2a. Impulse Response, Stable System

Figure 2b. Step Response, Stable System

Next consider the system , with system function

. The system has a single pole at z = 2 so the system is unstable. The impulse response and step response of the system are shown in Figures 3a and 3b. Note that the impulse response has infinite length and does not approach zero as n gets large. Also note that the step response grows without bound, a bounded input produces an unbounded output.

Figure 3a. Impulse Response, Unstable System

Figure 3b. Step Response, Unstable System

The impulse response and poles of the system indicate whether system is BIBO stable, a system’s stability does not depend upon the input to the system. A stable system can produce an unbounded output if the input is unbounded, for example a ramp input  may produce an unbounded output in a stable system.

## System Stability Examples

The system with transfer function

Is stable, both poles z = 0.81, -0.31 have magnitudes less than one. The impulse response is characterized by two decaying exponentials.

The system with transfer function

Is marginally stable, there is one pole z = -0.2 with magnitude less than one but there is one pole z = -1 on the unit circle in the z-plane.

The system with transfer function

Is unstable, there is one pole z = -0.2 with magnitude less than one but there is one pole z = -1.1 with a magnitude greater than one. The pole with a magnitude greater than one gives a term in the impulse response of , an increasing exponential.

## Frequency Response

The frequency response  of a stable discrete-time system is the discrete-time Fourier transform of the system’s impulse response. Here,  is used for discrete-time frequency;  is also used to represent discrete-time frequency.

The frequency response can also be obtained from the transfer function of a BIBO stable discrete-time system by substituting  in for z.

The frequency response can be expressed in terms of magnitude and phase functions

Where  is the gain of the system as a function of frequency and  is the phase shift of the system as a function of frequency.

The frequency response  of a discrete-time system is periodic with period , . Thus it is sufficient to know the frequency response over a one period interval, usually we use the Nyquist interval  rad.

## Frequency Response Examples

A two-point moving average filter  with transfer function  has frequency response is .

If the impulse response of the discrete-time system is not bounded, then the frequency response summation is a divergent series. For an unstable system, the frequency response is not meaningful even if one can determine it. For unstable systems, the sinusoidal response is drowned out by the increasing exponentials in the response.

The system  with system function  is a stable system. The frequency response is . A plot of the magnitude and phase portions of the frequency response of the system is shown in Figure 4a.

Figure 4a. Frequency Response Plot

The system  with system function  is an unstable system. The frequency response is not meaningful.

% system transfer function polynomials

Bz = [1, 0.5];

Az = [1, -0.5];

% check for stability

syspoles = roots(Az);

% frequency response, -pi to pi

wh = (-1:0.01:1)*pi;

H = freqz(Bz, Az, wh);

fig = figure;

subplot(2,1,1), plot(wh, abs(H));

ylabel('|H|'), grid, axis tight;

subplot(2,1,2), plot(wh, angle(H));

xlabel('wh (rad)'), ylabel('angle H'), grid, axis tight;

Figure 4b. MATLAB Code for Plotting Discrete-time Frequency Response

Last modified April 11, 2025

This work is licensed under CC BY-NC-SA 4.0. To view a copy of this license, visit https://creativecommons.org/licenses/by-nc-sa/4.0/