# Module 9 z-Transform

## z-Transform

The z-transform is a frequency domain transform (z domain) that is useful for discrete-time system analysis and design. The z-transform is convenient for mathematical analysis and synthesis of discrete-time systems. Applying the z-transform to sequences of values or weighted sums of impulses such as discrete-time signals x[n] and impulse responses h[n] results in polynomial functions X(z) and H(z) that are often easier to work with than the functions of n.

The bilateral or two-sided z-transform is defined as

Where z is a complex variable.

The unilateral or one-sided z-transform is defined as

The one-sided z-transform can be used when the function is a right sided signal (0 for n < 0).

The general form of the z-transform is an infinite sum and with the general form one must worry about whether the sum converges (finite sums will converge, infinite sums may or may not converge). The z-transform of x[n] exits if the signal increases no faster than exponentially

or  for some r

Almost all practical signals meet this criterion.

## Properties of the z-Transform

There are many useful properties of the z-transform for system analysis and design. The z-transform properties can be used with the z-transform pairs to determine transforms and inverse transforms for functions not in the tables.

### Linearity

The z-transform is a linear operator, i.e.

if   and , then

This allows a function with several components to be linearly decomposed into less complex terms and the z-transform done individually for each component. The z-transforms can then be summed to get the final result.

### Time Shifting (Delay)

If  then

If d > 0 then the sequence is shifted to the right (delayed) and if d < 0 then the sequence is shifted to the left (advanced),  corresponds to a unit delay.

### z-Domain Scaling (Multiplying by an Exponential)

If  then

### Multiplication by n (Differentiation)

If  then

### Time Reversal

If  then

### Initial and Final Value Theorems

Initial value theorem

Given a causal function x[n]

Final Value Theorem

Given a function x[n] that has a limit as

The final value theorem only applies if all the poles of  are inside the unit circle.

### Convolution

Convolution in the time domain corresponds to multiplication in the z-domain.

Multiplication in the time domain corresponds to convolution in the z-domain.

The convolution property is particularly useful for determining the system response of discrete-time systems.

## z-Transform Examples

The z-transform of a signal  using the z transform table:

and

The z-transform of a right-sided exponential  is

The z-transform of a delayed right-sided exponential

and then using linearity and the delay property the z-transform is

The z-transform of a finite duration exponential  is

Using the multiplication property requires convolution in z-domain and is not the best way to handle this.

and

## z-Transform of Finite Length Sequences

The z-transform of a signal represented as a sum of weighted impulses or a sequence of values can be performed term by term using the z-transform summation. This is only practical for relatively short sequences. Finite length discrete-time signals or sequences can be represented as a sum of weighted impulses and their z-transform found similarly.

The z-transform of  is

The z-transform of the sequence for n = 0 to 5 is

The z-transform of a pulse  using the z-transform summation

The z-transform of a finite length cosine signal using the z-transform summation

## Inverse z-Transform

The inverse z-transform takes a frequency domain (z-domain) function and transforms it to the time domain. The inverse z-transform is defined by

Where is integration around a closed path (contour integral) in the complex plane in a counterclockwise direction.

Generally, the inverse z-transform is not practical to compute using the inverse z-transform integral. As long as the inverse z-transform is restricted to being causal, the inverse z-transform is unique and

## Inverse z-Transform using Tables

Similar to the inverse Laplace transform, the inverse z-transform is relatively straightforward when the inverse z-transform can be determined using a combination of pairs and properties in the z-transform table. When the function of z is difficult to determine an inverse z-transform of using a table, the function of z can often be broken up into a sum of simper terms and then take the inverse z-transform of each of the terms.

If we have a function X(z) of the form  where each  can be found in the z-transform table, then one can find x[n] by taking the inverse z-transform of each term , , using the z-transform table. Direct division or partial fraction expansion (PFE) is often used when determining inverse z-transforms.

## Direct Division

Using direct division, X(z) is expanded into a power series in , in other words in the form.  The inverse z-transform can be done term by term using the delay property of the z-transform.

The drawback of this technique is that a closed form solution is usually not possible as the series is typically infinite in length.

## Partial Fraction Expansion (PFE)

A proper rational function of z, X(z), whose terms do not appear in the z-transform table can be expressed as a sum of fractions whose denominators are the poles of the original function and where each fraction appears in the z-transform table.

The rational function X(z) should have all common factors canceled out and should be of the form

For the partial fraction expansion, there may be poles in the following forms: real and distinct, real and repeated, complex and distinct (quadratic factors), and complex and repeated.

When the poles are all distinct, the partial fraction expansion will be of the form

Where the pk are the poles and the rk are the residues. The residue for a distinct pole can be found by canceling the pole and evaluating the remaining function at the pole (cover up method).

Complex poles will come in conjugate pairs (as long as polynomial coefficients are real) and the corresponding residues will be conjugates of each other.

From the partial faction expansion , the inverse z-transform is then taken using a z-transform table and is of the form .

Rational functions of z can be expressed as a ratio of polynomials in z and strictly proper means the degree of the numerator is less than degree of the denominator.

A partial fraction expansion on   can be performed to get an expansion in terms of positive powers of z. All of the examples here will be performed using the expansion in terms of negative powers of z.

## Partial Fraction Expansion Summary

Given a strictly proper rational function of z, :

Factor the denominator polynomial D(z) into a product of pole factors

Determine the residue for each pole factor,

The PFE is the sum of the residue divided by pole factor terms,

The inverse z-transform is then taken using the table of transform pairs,

## Inverse z-transform Examples

The inverse z-transform of

and  corresponds to a delay of five samples

The inverse z transform of

and

The inverse z transform of  requires a partial fraction expansion since X(z) is not typically found in a standard z-transform table

and

Then using the table to take the inverse z-transform term by term we obtain

The inverse z-transform of  also requires a partial fraction expansion

## PFE Using MATLAB

The roots of the polynomials and the residues can be found numerically using MATLAB. The roots function returns the roots of a polynomial. The residuez function returns the poles and residues assuming X(z) is expressed in terms of negative powers of z. The residue function (which is used for PFE on strictly proper rational functions of s) can also be used if X(z) is expressed using positive powers of z and a partial faction expansion of X(z)/z is desired.

When working with polynomials, the polynomial terms are arranged highest order term to left and lowest order term to the right. With negative powers of z that means the constant term is the leftmost term and the highest negative power of z is the rightmost term. This is important for correctly entering the polynomial coefficients in MATLAB for determining roots and performing partial fraction expansions.

The MATLAB code segment of Figure 1a will determine the residues and pole factors of the X(z) function below. The result of the residue function is shown in Figure 1b.

% coefficients of B(z) and A(z)

Bz = [7, -3.6];

Az = [1, -0.6, -0.16];

[r,p,k] = residuez(Bz,Az);

Figure 1a. MATLAB Code Segment for Partial Fraction Expansion

r =

2

5

p =

0.80000

-0.20000

k =

[]

Figure 1b. Result from residuez Function in Code of Figure 1a

Where r is a row vector of the residues, p is a row vector of the poles, and k is the coefficients of the quotient polynomial if the function is not proper.

## Partial Fraction Expansion, Not Strictly Proper

If the function  is not proper, it can be put in the form  where  is proper by dividing A(z) into B(z) and expressing X(z) as a quotient and remainder. The remainder will be proper and a partial fraction expansion can be performed on it.

Consider a rational function of z that is not strictly proper

, so X(z) is a proper (but not strictly proper) rational function of z. If  then we have an improper rational function of z.

Where,  and  are found by dividing the denominator into the numerator. PFE can then be done on the strictly proper rational function .

The MATLAB code segment of Figure 2a will determine the residues and pole factors of the X(x) function below which is not strictly proper. The result of the residue function is shown in Figure 2b.

% coefficients of B(z) and A(z), not strictly proper

Bz = [8, -2.4, 0.6];

Az = [1, -0.5, 0.06];

[r,p,k] = residuez(Bz,Az);

Figure 2a. MATLAB Code Segment for Partial Fraction Expansion

r =

19.99999

-21.99999

p =

0.30000

0.20000

k =

10

Figure 2b. Result from residuez Function in Code of Figure 2a

## Symbolic z-Transforms and Inverse z-Transforms using MATLAB

MATLAB’s symbolic toolbox has functions for performing z-transforms, ztrans, and inverse z-transforms, iztrans, on symbolic functions of n and z. The result of the ztrans function is in terms of positive powers of z.

The MATLAB code segment of Figure 3 determines the z-transform of  symbolically.

% symbolic z-transform

sympref('HeavisideAtOrigin', 1.0);

syms n z;

xn = 2.5*(0.1)^n*heaviside(n);

Xz = ztrans(xn,n,z);

disp(Xz);

Figure 3. MATLAB Code Segment for Determining Symbolic z-Transform

The result is 5/(2*(10*z - 1)) + 5/2, which is equivalent to

The MATLAB code segment of Figure 4 determines the inverse z-transform of . The functions of can be expressed using negative or positive powers of z.

% symbolic inverse z-transform

Xz = (1 + (1/z))/(1 - 1.5*(1/z) + 0.5*(1/z)^2);

% Xz = (1 + z^(-1))/(1 - 1.5*z^(-1) + 0.5*z^(-2));

% Xx = (z^2 + z)/(z^2 - 1.5*z + 0.5);

xn = iztrans(Xz,z,n);

disp(xn);

Figure 4. MATLAB Code Segment for Determining Symbolic Inverse z-Transform

The result is 4 - 3*(1/2)^n, which is equivalent to

The symbolic operations results are often in forms that are different from what a person would get using tables and are not always simplified well.

Last modified April 10, 2025

This work is licensed under CC BY-NC-SA 4.0. To view a copy of this license, visit https://creativecommons.org/licenses/by-nc-sa/4.0/