Lab Actvity: Traffic Flow and Density Lab

1) Go to this website: https://traffic-simulation.de/ring.html

This is a simulation of traffic flowing around a circular track. There are three lanes in the circular track. Along the circular track, there are four sensors placed above the road which measure “Flow”, “Speed” and “Dens” (density) of vehicles crossing the sensor. The readings of “Flow” and “Speed” from the four sensors will be used in determining the average “Flow” and “Speed” of the traffic in the simulation. The average values of “Flow” and “Speed” will be reported in the Table-1.

As soon as the web page is loaded, the simulations starts with parameters mentioned on the left half of the web-page. The simulation may be paused by clicking the red “Pause” button. After pausing the simulation, the various sliders on the right half of the webpage may be used to choose the simulation parameters.  The green refresh button (circular arrow) is used in restarting the simulation with new parameters. In this lab, we will only be varying two parameter sliders, they are:

- Density/lane

- Truck Perc

2) Using the slider “Truck Perc”, make “Truck Perc” to 0% as we don’t need trucks in this study

3) The slider “Density/lane” is used to vary density to perform simulations for various densities listed in Table-1

4) Note: whenever you change the “Density/lane”, make sure to click on the green refresh button, and then measure

5) Fill the Table-1 below. For flow and speed, calculate the average values from the four measurements at each density corresponding to the 4 sensor locations

| Density (y-axis, step-7) | Average Speed (km/h) (y-axis, step-6) | Average Flow (veh/h) (x-axis) |
| --- | --- | --- |
| 1 |  |  |
| 10 |  |  |
| 20 |  |  |
| 30 |  |  |
| 40 |  |  |
| 50 |  |  |
| 60 |  |  |
| 70 |  |  |
| 80 |  |  |
| 90 |  |  |
| 100 |  |  |

6) In excel, draw a scatter plot of speed (y-axis) vs density (x-axis). Comment on how does the average speed depend on the average flow. Explain briefly as to why your finding is reasonable.

7) In excel, draw a second scatter plot of Flow (y-axis) vs density (x-axis). Comment on the shape – is it increasing or decreasing?

8) From scatter plot in step-7, comment on why does the “Flow” versus “density” plot show a variation that you observe? How can you explain the observed scatter plot behavior in step-7?

9) In step-7 plot, at what density do you start observing traffic jams?

10) In step-7 plot, at what density is the flow maximum? Comment on a reason why is this so?